Veneer Most species – resawn 3 veneers from one 4/4 board

| First Pass | .200 |  |
| --- | --- | --- |
| Second Pass | .165 | All veneers must be consistent |

1 3/4" Rails glued for width

| First Pass | .930 |  |
| --- | --- | --- |
| Second Pass | .905 |  |

Rough Stiles – After the edges have been glued onto the core

| Door Thickness | 1 3/8” | 1 3/4” | 2 1/4” | 2 1/2" |
| --- | --- | --- | --- | --- |
| Top Head | 1.095 | 1.470 | 1.970 | 2.225 |
| Bottom Head | .260 | .260 | .260 | .260 |

Stiles, Rails, and TDL material

| Description | # of pieces | Top Head | Bottom Head |  |
| --- | --- | --- | --- | --- |
| Stiles | 1 | 1.150 | .260 |  |
| 1 3/8” rails | 2 | .720 | .160 |  |
| 1 3/8” TDL | 2 | .850 | .050 |  |
| Stiles | 1 | 1.525 | .260 |  |
| 1 3/4” rails | 2 | .905 | .020 | 1 3/4" rails glued for width send through veneer sander |
| 1 3/4” rails 2+1 veneer |  | .827 | .060 |  |
| 1 3/4” TDL | 3 | .650 | .200 |  |
| Stiles | 1 | 2.025 | .260 |  |
| 2 1/4” rails | 3 | .770 | .125 |  |
| 2 1/4” TDL | 3 | .813 | .060 |  |
| Stiles | 1 | 2.280 | .260 |  |
| 2 1/2" rails | 3 | .855 | .040 |  |
| 2 1/2” TDL | 3 | .900 | .020 |  |